MANDATORY OWNER CHANGE  /  DESIGN BASIS  /  R08 → R09

# ARC-1 COMPACT

# R08 Corrective Engineering Handoff

NOR-FIRST BOOT  |  SD GAME LIBRARY  |  USB DEVICE-ONLY  |  FPGA BUS ISOLATION

Document control: v1.0  •  Prepared 15 October 2026  •  Audience: Claude (R08 electrical / firmware engineer) and R09 PCB integrator

OWNER ORDER Modify the R08 work already underway. Do not restart it. Do not silently substitute SD boot for NOR boot. The exact LCD is a separate R09 decision. This document supersedes conflicting older R07/R08 notes on storage, boot priority, USB-host support and display selection.

Figure 1. Required ownership of boot, games, graphics and diagnostics. Functional diagram—not a verified pin-level schematic.

# Decision register

| ID | Owner decision | Engineering meaning |
| --- | --- | --- |
| D-01 | Onboard 32 MiB (256 Mbit) NOR | Required for the first PCB; exact manufacturer/package pending validation. |
| D-02 | Boot with no SD inserted | Load startup, graphics drivers and launcher wholly from NOR. |
| D-03 | microSD is game media | Games, art/music, game configuration and saves; absent card is normal. |
| D-04 | No USB thumb-drive host | Retain USB-C power, USB0 device diagnostics and ROM/FEL access. |
| D-05 | F133 → MN820 direct | Graphics remain operational without an FPGA bitstream. |
| D-06 | T35 optional accelerator | Retain T35/8 MiB PSRAM, preserve native FPGA verification. |
| D-07 | Exact screen in R09 | No premature LCD, FFC footprint or LED driver freeze. |

# 1  Why R08 must be interrupted

Adding permanent NOR changes startup pin states, board-level boot selection, SPI0 ownership, manufacturing programming and fault recovery. These are schematic-level choices; attempting them after R09 routing would create avoidable rework. Conversely, selecting an LCD now is unnecessary. R08 shall finish the common LVDS/graphics architecture but explicitly carry panel-dependent circuitry as an R09 dependency.

# 2  Verified technical anchors versus open questions

| Evidence type | Supported finding | What is NOT yet proven |
| --- | --- | --- |
| F133 manual [S1] | GPIO selection 00 chooses SPI NOR before SPI NAND when BOOT_MODE selects GPIO. | Whether selected F133-B samples GPIO or eFuse in its delivered state; board reset strap design. |
| F133 pin mux [S2] | PC2=SPI0_CLK; PC3=SPI0_CS0; PC4=SPI0_MOSI/BOOT-SEL0; PC5=SPI0_MISO/BOOT-SEL1; PC6/7=SPI0_WP/HOLD. | No contention across NOR, FPGA, reset, flash power state and strap sampling. |
| Winbond [S3,S4] | W25Q256JV family: 256 Mbit, 3.3 V class, 3/4-byte addressing. | Boot-ROM support of chosen exact part, 4-byte access by loaded software, stock/lifecycle. |
| Allwinner FEL [S1,S5] | USB FEL is a boot-ROM concept; USB0 is the F133 dual-role physical interface. | Exact accessible forced-FEL condition, recovery tool flow and blank-board programming. |
| Bridgetek [S6] | MN820 integrates BT820B and 1 Gbit (128 MiB) DDR3L, 3.3 V module input. | Any panel-specific connection or final backlight design. |
| Efinix [S7] | T35 needs validated supplies, reset, pin constraints, FPGA configuration. | Completed native placement/timing/bitstream and proven no-drive power-up behavior. |

# 3  Non-negotiable power-on behavior

Normal, SD absent: Cold power-on reads NOR, initializes system RAM, talks directly to MN820, and reaches an empty-library launcher. No FPGA configuration is necessary.

Normal, SD present: The same NOR system starts first; subsequent card detection/mount reads available game packages. A game card shall not replace system firmware.

NOR blank or corrupt: A user-accessible independent ROM/FEL programming route allows diagnosis and reprogramming without executing NOR-resident code.

T35 failed or unconfigured: Boot, graphics and USB recovery remain possible, provided the defective FPGA is non-driving on any shared bus.

SD removed during gameplay: Stop new card I/O and return to the launcher safely where possible; reject incomplete saves rather than corrupting data silently.

# 4  F133 boot ROM, SPI0 and boot straps — blocking design issue

The F133 manual describes both GPIO and eFuse boot-source selection. Under GPIO selection, BOOT-SEL[1:0]=00 prioritizes SPI NOR. The previous 11 strap concept targeted SD/eMMC and must not remain unquestioned. PC4/PC5 are shared between boot selection and SPI0 data; the strap network must reliably establish boot priority at reset while permitting subsequent SPI signaling. [S1,S2]

| Pin | Multiplexed function | R08 mandatory review |
| --- | --- | --- |
| PC2 | SPI0 clock | F133-to-NOR wiring, signal integrity, FPGA clock input isolation. |
| PC3 | SPI0 chip select 0 | NOR CS# selection during BROM; no second slave active at reset. |
| PC4 | SPI0 MOSI / BOOT-SEL0 | Correct low strap at sampling; inspect flash/FPGA leakage and pull conflicts. |
| PC5 | SPI0 MISO / BOOT-SEL1 | Correct low strap at sampling; ensure NOR and T35 cannot drive at sample time. |
| PC6 | SPI0 WP#/IO2 | Check quad-mode requirements and reset pull; no unintended output. |
| PC7 | SPI0 HOLD#/IO3 | Check quad-mode requirements and reset pull; no unintended output. |

HARD GATE Do not declare NOR boot valid from a GPIO table alone. Provide the verified CPU boot-source selection method, logic levels/timing, resistor values, exact SPI net ownership and a boot-time state table. If FPGA communication must move or require hardware isolation, correct it in R08.

## 4.1  Bus-ownership state matrix

| Phase | F133 SPI0 | NOR | T35 / other shared nodes |
| --- | --- | --- | --- |
| Power ramp | Reset / uncontrolled until POR | CS# held inactive; supply stable | Held in reset; verify safe I/O, no back-power. |
| Strap sampling | PC4/PC5 sampled for boot | MISO must not overpower strap | No active driver or opposing pull on sampled pins. |
| BROM read | Only intended master | Selected and readable; safe opcode/address | CS deasserted and I/O inputs; no bus contention. |
| Application | NOR flash access as needed | Valid nonvolatile image | May communicate on qualified CS, with reset/enable sequencing. |
| USB FEL | Host recovery independent of firmware | May be blank/corrupt | Must stay disabled or electrically noninterfering. |

The T35 reset assertion alone is not accepted as proof of all pad states. Check the exact T35F400C3 package, I/O bank supplies and Efinix pin-state tables under reset, unconfigured and configured states. Consider explicit output-enable gating when required. The same applies to unpowered FPGA back-feed through SPI signals. [S7]

# 5  Mandatory 32 MiB NOR: capacity, addressing, component qualification

Owner storage target = 32 MiB = 256 Mbit = 33,554,432 bytes. W25Q256JV is a reference family, not a frozen BOM line. A particular Winbond variant can be marked “not recommended for new designs”; select by an exact orderable suffix, not the generic W25Q256JV label. Winbond parts at this density may power up in 3-byte address mode and need extended-address handling above the first 16 MiB. [S3,S4]

Boot0 / boot-ROM prefix — Show exact offset, ROM-recognized image/header, read opcode, address width, size bound, power-up address mode and valid first-stage load path. Keep ROM-required startup data in the reachable low-address region.

Full device addressing — Prove how the loaded driver addresses the second 16 MiB (0x01000000 and above). Include JEDEC/SFDP detection, 4-byte instructions or bank-register strategy and reset-mode safety.

Image-size ledger — Measure the actual boot, system, graphics/audio drivers, launcher, FPGA diagnostic resources, recovery and update images. Do not silently assume Linux fits in 32 MiB or 64 MiB system RAM.

Programming and lifecycle — Provide first-assembly programming (FEL or external programming fixture), update/rollback or recoverable update strategy, verify/readback, write-protect and status-bit configuration.

Parts and footprint — Choose exact P/N, package, vendor status, voltage, JEDEC ID, QE/reset options, sourcing and JLCPCB assembly feasibility. SOIC16 vs WSON8 vs BGA has different diagnostic tradeoffs.

## 5.1  Illustrative flash budget — allocate after measuring real binaries

| Partition concept | Illustrative MiB | Engineering restriction |
| --- | --- | --- |
| Boot image + redundant metadata | 2 | Boot-ROM layout dictates actual placement; avoid single point of power-fail corruption. |
| Primary system image | 9 | Measured binary + manifest; launcher and drivers remain self-contained. |
| Secondary update slot | 9 | Use only if measured images fit; otherwise document alternative. |
| Minimal recovery image | 4 | Recovery must not be required to enter USB ROM/FEL. |
| Built-in assets / FPGA fallback | 4 | Only boot-critical assets. Game data stays on microSD. |
| Settings, integrity journal | 1 | Wear leveling / redundant sectors, power-failure-safe metadata. |
| Spare / growth | 3 | Subject to approved flash layout and erase-block geometry. |
| TOTAL | 32 | Example arithmetic only; NOT a mandated partition map. |

ADDRESSING TRAP 24-bit addresses cover only 16 MiB. A 32 MiB device requires explicit planning for how later firmware reaches the upper half. A successful first boot from offset zero does not validate the whole flash device.

# 6  USB-C: power sink and diagnostic device, not thumb-drive host

Remove USB mass-storage HOST requirements, USB VBUS sourcing to accessories and host-only protection/power-switch paths. Do not remove the CPU’s USB0 device data connection or the ability to enter ROM/FEL. The F133 documentation distinguishes USB0 dual-role from USB1 host. [S1,S2]

| Preserve | Remove or defer |
| --- | --- |
| USB-C 5 V sink path, CC1/CC2 sink termination / safe current budget | USB-A receptacles and flash-drive enumeration |
| F133 USB0 D+/D− with ESD, approved routing and connector | USB host VBUS boost/current switch for flash drives |
| Unbooted-board FEL entry, reset access, PC-side flashing tools | USB keyboard, HID host and wired game-networking requirements |
| USB debug service when firmware is running | Any promise of laptop-port full-load power without current analysis |
| Optional battery charging only if battery adopted | Unneeded USB PD / display-alt-mode complexity |

Require a specific recovery procedure: power from a safe source, hold a documented force-FEL action, assert/release reset, observe USB ROM enumeration, upload a small SRAM stub and program/verify NOR. Board provisions must support this even with NOR erased. USB Type-C CC implementation, allowable sink current, backfeed prevention and battery/USB power-path arbitration require review. Generic sunxi FEL guidance is not a substitute for proving the exact F133 boot entry and package signal. [S1,S5]

# 7  microSD: removable games, not system boot

Use the F133-supported SD host interface with verified mux and signal voltage. Card detect, ESD, power switching where needed, insertion/removal signaling, filesystem and write-safe saves must be documented. An empty card slot is a normal startup condition. A malicious/corrupted game package should not prevent the NOR launcher from booting.

Media interface proof — Map SMHC pins, DAT0–DAT3, CLK, CMD, detect, VDD, pull resistors, I/O levels and power sequence. Avoid reusing conflicting SPI0/boot pins.

Runtime behavior — Scan a versioned game manifest; reject invalid packages; distinguish card absence, corruption and unsupported format; journal or atomic-rename saves where suitable.

Test matrix — Boot with no card, empty card, valid games, faulty filesystem, insertion during launcher, removal during save, sudden power loss.

# 8  Preserve graphics, FPGA, PSRAM, audio and power architecture

| System | Locked direction | R08 validation focus |
| --- | --- | --- |
| F133-B | 64 MiB in-package DDR2; CPU runs game/system firmware | Pin mux, NOR boot stages, SDRAM startup, UART log and watchdog. |
| MN820 / BT820B | F133-to-BT820 direct SPI path; 128 MiB on-module DDR3L | CPU SPI1 connectivity, module 3.3 V input direction, reset, interrupt, logic thresholds. |
| T35F400C3 | Optional hardware accelerator | Native Efinity project, legal banks/pin map, configuration delivery, reset/tri-state. |
| 8 MiB QSPI PSRAM | T35 local working-data memory | VDD, boot-time CE#, CLK/IO idle levels, clock/IO routing and timing. |
| Two MAX98357A | F133 digital stereo through two I²S amplifiers | F133 pin mux, channel-select straps, 2.5–5.5 V supplies, speaker connector. |
| Power | USB-C sink; battery if adopted | Rail sequencing, 3.3 V/1.x V domains, reset supervision, worst-case demand. |

MN820 integrates BT820B plus its own 1 Gbit DDR3L and accepts 3.3 V module input; never presume a module pin is a safe output or input without its exact pin table. MAX98357A supports I²S stereo channel selection when two parts are used. [S6,S8]

## 8.1  Scope boundary for display

R09-DEPENDENT R08 shall preserve MN820 LVDS differential output mapping, the physical module connector, safe related GPIO/control assignments and a power/backlight allowance. R08 shall NOT freeze panel model, pixel clock, FFC footprint, pinout, bezel, LCD supply or backlight driver. R09 must revise the schematic/ERC after panel selection and BEFORE PCB routing is called finished.

# 9  R08 evidence hierarchy and debug gates

| Gate | Required evidence | Failure response |
| --- | --- | --- |
| G0 Owner alignment | Decision log reflects all seven decisions and explicit LCD deferral. | Stop: rework spec. |
| G1 Boot electrical | Documented boot select/eFuse source, straps, reset net, NOR nets, pin mux. | Stop: NOR/FPGA conflict unresolved. |
| G2 Chip qualification | Exact flash vendor P/N, geometry, voltage, address-mode strategy, sourcing. | Stop: unsupported boot/RAM budget. |
| G3 Recovery proof | Hardware path for USB0/FEL plus blank-NOR PC host procedure. | Stop: no reliable unbooted programming. |
| G4 Native CAD | KiCad .kicad_sch and actual ERC/netlist; reference/footprint check. | Do not substitute PDFs/notes. |
| G5 FPGA native | Efinity constraints, synth/place-route/timing report or explicit blocker. | Label unverified; do not claim 100% success. |
| G6 Firmware | Compile/run test harness, manifest, negative tests; tests labeled simulation. | No physical-boot claims. |
| G7 Power / assembly | Worst-case rail budget, USB sink, power sequence, part availability. | R09 prerequisite / blocker as appropriate. |
| G8 R09 intake | Open issues with owner, due revision, exact next validation. | No “fabrication-ready” verdict. |

Evidence categories must be kept distinct: (A) manufacturer-published facts, (B) electrical calculations, (C) native CAD/EDA tool reports, (D) software simulation and (E) actual physical hardware observations. R08 can provide A–D. No one has yet performed E on an ARC-1 production PCB.

# 10  Required R08 deliverables to hand to R09

Engineering change summary: Exact old-to-new schematic net, component and code changes; preserved work; removed USB host and SD-boot assumptions.

Native schematics: KiCad source, plot PDFs, ERC output, netlist, pin table and all references/footprints with version-control commit.

Boot trace dossier: Cold-start state diagram; BOOT_MODE/eFuse evidence; resistor/CS math; NOR first 16 MiB access; selected flash ID and configuration.

Flash image dossier: Measured firmware image sizes, boot0 offsets, lower/upper half tests, full capacity tests, boot fallback/update scheme.

Recovery dossier: F133 USB0 and FEL entry path, connector hardware, PC commands and first-board NOR flashing/readback procedure.

FPGA dossier: Verilog, native Efinity version/license, pin-bank constraints, psram controller proof, timing/bitstream or explicit blockers.

Power/audio dossier: Named rails, current and sequencing budget, 2x amp I²S proof, USB battery/charging decisions, test points.

R09 open-issues manifest: LCD/panel/connector/backlight, enclosure/PCB outline, final SI/power layout, verified purchasing and any unresolved electrical risks.

# 11  R08 completion is NOT manufacturing approval

R08 completion means the core electrical architecture is internally consistent, represented in native engineering sources, and thoroughly checked to the extent possible without hardware. It does not mean a board has been routed, fabricated, powered or successfully booted. R09 will select LCD and mechanical constraints and route the PCB; R10 should independently audit the final design before a first JLCPCB order.

# 12  Paste-ready instruction for Claude (owner-authoritative)

Integrate this attached ARC-1 COMPACT R08 Corrective Engineering Handoff v1.0 into the R08 project already underway. Do not restart completed valid work. Mandatory: soldered 32 MiB SPI NOR and NOR-independent boot (even with SD absent); verify F133 GPIO/eFuse boot selection and SPI0/PC4–PC5 FPGA contention; select an exact supportable flash and handle 24/32-bit addressing; preserve independent USB0/FEL entry and first-assembly reflash; microSD for games/assets/saves only; remove USB thumb-drive HOST support; retain direct F133→MN820 graphics, optional T35 with 8 MiB PSRAM, and stereo I²S amplifiers. The exact LCD, screen connector and backlight circuit are postponed until R09, and must remain explicitly open. Deliver updated native KiCad/firmware/FPGA sources, pin maps, boot-state table, recovery procedure, source-backed electrical evidence and clear unresolved blockers. Never call this a completed or fabrication-ready PCB without native evidence. Stop and ask if any of these requirements are electrically impossible.

# 13  Source register (primary sources preferred)

[S1] Allwinner F133 User Manual, Rev 1.0, §3.4 BROM, pp. 144–150.
https://www.mangopi.org/_media/f133_user_manual_v1.0.pdf

[S2] Allwinner F133 Datasheet, pin multiplexing and USB0/USB1 distinction.
https://bbs.aw-ol.com/assets/uploads/files/1639798962951-f133_datasheet_v1.0_whycan.pdf

[S3] Winbond W25Q256JV SPI datasheet, 256 Mbit address modes, instruction set, JEDEC, power. Manufacturer documentation.
https://www.winbond.com/hq/support/documentation/?__locale=en&category=%2F.categories%2Fresources%2Fdatasheet%2F&family=%2Fproduct%2Fcode-storage-flash%2Fqspi-nor%2Findex.html&line=%2Fproduct%2Fcode-storage-flash%2Findex.html&pno=W25Q256JV

[S4] Winbond exact-variant product status; W25Q256JVEIQ has N=new-design status caution.
https://www.winbond.com/hq/product/code-storage-flash/qspi-nor/w25q-jv/?__locale=en&partNo=W25Q256JVEIQ

[S5] U-Boot sunxi documentation: USB OTG FEL recovery concepts, PC enumeration, tools.
https://docs.u-boot.org/en/latest/board/allwinner/sunxi.html

[S6] Bridgetek MN820 product page and DS_MN820 Rev 1.0.
https://brtchip.com/product/mn820/

[S7] Efinix Trion documentation center; T35 Data Sheet v3.6 latest listed, pin/configuration reference.
https://www.efinixinc.com/support/docs.php?p=t

[S8] Analog Devices MAX98357A production I²S class-D amplifier specifications.
https://www.analog.com/en/products/max98357a.html

Sources accessed during the preparation of this handoff on 15 October 2026. Source-document availability and vendor revisions must be reconfirmed before component ordering. This document is a controlled engineering instruction, not proof that the referenced PCB has been electrically or physically validated.